TITULO: EVALUACION DE ESCALAS ANALITICA-PRODUCTIVA EN LA PURIFICACION DEL ANTIGENO DE LA HEPATITIS B.

Autores: Ramcy Regalado Hernández1, Galina Moya Fajardo2, Miguel Castillo Ferrer3.

Correo: ramcyrh@gmail.com

Afiliación: 1Centro de Ingeniería Genética y Biotecnología, La Habana, Cuba

2 Centro de Ingeniería Genética y Biotecnología, La Habana, Cuba

3 Centro de Ingeniería Genética y Biotecnología, La Habana, Cuba

Resumen:  Introducción: La purificación del antígeno de la Hepatitis B es fundamental en la producción de vacunas recombinantes como Heberbiovac. La transferencia tecnológica (analítica-productiva) requiere homologar parámetros como la capacidad de elución y % de recobrado. Objetivo: Evaluar de forma comparativa las escalas de purificación del antígeno por Cromatografía de Inmunoafinidad, identificando factores que afectan su equivalencia y optimizando los parámetros a pequeña escala para mejorar el proceso cromatografico. Materiales y Métodos: Se utilizó una matriz de Sepharosa con anticuerpo monoclonal CB.HEP-1 de ratón. Los primeros experimentos se realizaron en una columna analítica de 5 cm de altura y 3,93 ml y una matriz de producción de 30 cm de altura y 16 L. En ambas se aplicó el mismo antígeno a un flujo lineal constante de 150 cm̸h y un tampón fosfato de pH 7. Se calcularon la capacidad de elución y el % de recobrado. En la etapa de optimización a escala analítica se realizaron 7 experimentos incrementando la altura del gel y disminuyendo el tiempo de residencia del antígeno. Resultados: Los resultados de recobrado y capacidad de elución difieren: 44,99 % (Laboratorio) contra 39,38 % (Producción), p=0,023, capacidad productiva (0,041) y capacidad analítica (0,065). Conclusiones: No se logra la homologación, pero se identificaron como factores críticos: el tiempo de aplicación del antígeno, saturación de la matriz y la degradación del ligando. Cambiar el tiempo de residencia en laboratorio mejoro la capacidad hasta 0,107 mg y el recobrado con 49,29 % mostrando un avance en la optimización del proceso de purificación.

Palabras claves: matrices cromatografías, Cromatografía de inmunoafinidad anticuerpos monoclonales, ligando, capacidad de elución, % de recobrado, antígeno.

TITLE: EVALUATION OF ANALYTICAL-PRODUCTIVE SCALES IN THE PURIFICATION OF HEPATITIS B ANTIGEN.

Authors: Ramcy Regalado Hernández1, Galina Moya Fajardo2, Miguel Castillo Ferrer3.

Email: ramcyrh@gmail.com

Affiliation: 1Center for Genetic Engineering and Biotechnology, Havana, Cuba

2 Center for Genetic Engineering and Biotechnology, Havana, Cuba

3 Center for Genetic Engineering and Biotechnology, Havana, Cuba

Summary: Introduction: The purification of the Hepatitis B antigen is fundamental in the production of recombinant vaccines such as Heberbiovac. Technological transfer (analytical-productive) requires standardizing parameters such as elution capacity and % recovery. Objective: To comparatively evaluate the antigen purification scales by Immunoaffinity Chromatography, identifying factors that affect their equivalence and optimizing small-scale parameters to improve the chromatographic process. Materials and Methods: A Sepharose matrix with the mouse monoclonal antibody CB.HEP-1 was used. The initial experiments were carried out in an analytical column 5 cm high and 3.93 ml, and a production matrix 30 cm high and 16 L. In both, the same antigen was applied at a constant linear flow of 150 cm/h and a pH 7 phosphate buffer. The elution capacity and % recovery were calculated. In the optimization stage at the analytical scale, it was carried out 7 experiments were conducted by increasing the height of the gel and decreasing the residence time of the antigen. Results: The recovery and elution capacity results differ: 44.99% (Laboratory) versus 39.38% (Production), p=0.023, productive capacity (0.041) and analytical capacity (0.065). Conclusions: Standardization is not achieved, but critical factors were identified: the antigen application time, matrix saturation, and ligand degradation. Changing the residence time in the laboratory improved the capacity up to 0.117 mg and the recovery to 49.29%, showing progress in the optimization of the purification process.

Keywords: chromatographic matrices, monoclonal antibody immunoaffinity chromatography, ligand, elution capacity, % recovery, antigen.